This doco. supports the file called RCSRADIO.ETL  (A DRILL SIZE TOOL TABLE)

RCSRADIO.ETL is a commonsense TOOL TABLE for within the use of the EASYPLOT.

This file was generated because of the very poor ridiculous suggested drill

sizes recommended by the guru's from PROTEL.   (who dunno wot a pcb is)

The suggested TOOL TABLE Drill Sizes relevant to the typical PAD sizes are

larger in this file so as to allow for the removal of components and

re-insertion of replacement components due to failure (sometimes).

To change any typical drill size I suggest that you copy the RCSRADIO.ETL file

into another directory and RE-NAME it to eg. YOURNAME.ETL then MOVE it back to

the EASYTRAX directory and by using any sort of ASCII Editor you can nominate

any drill size against any TOx (eg. TO1, TO2, TO3, TO4, TO5, TO6, TO7, TO8)

The suggested Drill sizes are as follows

PAD         TOOL  Thou     rcs comments only and examples of typical use.

ROUND40     TO1    32      or o.8mm very very small ident pads only

ROUND50     TO1    32      or o.8mm very small ident pads only

ROUND62     T02    36      or o.9mm IC Pads or Small Transistor Pads

ROUND70     TO3    4O      or 1.omm Normal Resistors, Diodes.

ROUND85     TO3    4O      or 1.omm Normal most components PADS etc.

ROUND100    TO4    48      or 1.2mm 5watt resistors etc

ROUND125    TO5    6O      or 1.5mm larger TO22O SCR's and legs etc.

ROUND150    TO6    8O      or 2.omm as in very large pins, fuse clips etc.

ROUND175    TO7   100      or 2.5mm hard wire I/O connections etc.

ROUND200    TO7   100      or 2.5mm hard wire I/O connections etc.

ROUND225    TO8   125      or 3.2mm earths and mounting holes etc.

ROUND250    TO8   125      or 3.2mm 2N3O55 can connections and TO22O common

this gap not normally in the RCSRADIO.ETL TOOL TABLE

SQUARE40    TO1    32      or o.8mm very very small ident pads only

SQUARE50    TO1    32      or o.8mm very small ident pads only

SQUARE62    T02    36      or o.9mm IC Pads or Small Transistor Pads

SQUARE70    TO3    4O      or 1.omm Normal Resistors, Diodes.

SQUARE85    TO3    4O      or 1.omm Normal most components PADS etc.

SQUARE100   TO4    48      or 1.2mm 5watt resistors etc

SQUARE125   TO5    6O      or 1.5mm larger TO22O SCR's and legs etc.

SQUARE150   TO6    8O      or 2.omm as in very large pins, fuse clips etc.

SQUARE175   TO7   100      or 2.5mm hard wire I/O connections etc.

SQUARE200   TO7   100      or 2.5mm hard wire I/O connections etc.

SQUARE225   TO8   125      or 3.2mm earths and mounting holes etc.

SQUARE250   TO8   125      or 3.2mm 2N3O55 can connections and TO22O common

this gap not normally in the RCSRADIO.ETL TOOL TABLE

R40X120     TO1    32      or o.8mm very very small ident pads only

R50X120     TO1    32      or o.8mm very small ident pads only

R60X120     T02    36      or o.9mm IC Pads or Small Transistor Pads

R70X120     TO3    4O      or 1.omm Normal Resistors, Diodes.

R80X120     TO3    4O      or 1.omm Normal most components PADS etc.

R90X120     TO4    48      or 1.2mm 5watt resistors etc

this gap not normally in the RCSRADIO.ETL TOOL TABLE

DIP40       TO1    32      or o.8mm very very small ident pads only

DIP50       TO1    32      or o.8mm very small ident pads only

DIP60       T02    36      or o.9mm IC Pads or Small Transistor Pads

DIP70       TO3    4O      or 1.omm Normal Resistors, Diodes.

DIP80       TO3    4O      or 1.omm Normal most components PADS etc.

DIP90       TO4    48      or 1.2mm 5watt resistors etc

this gap not normally in the RCSRADIO.ETL TOOL TABLE

VIA40       TO1    32      or o.8mm Very small via - data only

VIA50       TO1    32      or o.8mm Very small via - data only

VIA62       TO2    36      or o.9mm Normally the smallest via - data only

VIA70       TO3    4O      or 1.omm Normal via's data and small current

VIA100      TO4    48      or 1.2mm use in tripple bus conns for power supply

A hint is when using via's is to try and put the largest VIA on the board so

as maximum current can be sent down  through  the  lining  of the via hole.

I suggest you use the VIA70 for most data and the VIA100 in a Double or Triple

placement on the track for maximum track to track reliability.

Multiple via's are cheaper than repairing a board with blown via's due to

current overload - due to poor design by the CAD operator.

end of file.

-----------------------------------------------------------------------------

To call up the new DRILL TOOL TABLE when in EASYPLOT main menu do the following

S etup - this puts you in the setup (alter menu)

N C Drill - this now puts you in the NC DRILL SETUP menu

(The Output file is only if you are making a stand alone file so)

(go down to the the label called Tool Table)

Tool Table : [ENTER]

at this point edit/repair the Drive No. and sub directory and the

file to be used eg.

A TOOL TABLE Menu will come on and list any *.ETL files

so scroll down and select the new *.ETL of your choice

either STANDARD.ETL - If you are nuts

RCSRADIO.ETL - If you are smarter

yourname.etl - If you are game - on your head be it.

C:\ET\RCSRADIO.ETL - this now loads the better Tool Table on line.

So upon selecting and then pressing <ESC> it will save thus

next hit <ESC> again back to the SETUP MENU, then the main menu,

and to save this as normal upload table bail out by F ile  Q uit.

Thus uploads the new TOOL Table for manufacturing.

Usual backtracking after using the S (setup) N (NC DRILLING) Library.

Use this instead of the STANDARD.ETL (EASYTRAX DRILL SIZE LIBRARY)

Comments on the RCSRADIO.ETL are welcome, you can tele-call me on

+61 2 587-3491 and ask for Robert J Barnes, or shower me in $5.oo

notes for saving you a bundle on your next production from your

favourite manufacturer owing to PROTEL's misguided drill file.

This file is not copy write - please distribute in the interests

of producing more manufacturing friendly artworks.