Multigame / Single rom hack for Galaxian PCB

--------------------------------------------

program the files into the following roms

fourplay.bin into a 27512 rom

graph1 and graph2 into 2732 roms

then follow the following steps

1. Remove rom board and Graphics roms.

2. On a 28 pin socket (or the 27512 programmed earlier) bend out

pins 1,2,20,22,23,26,27 and 28

3. Insert socket (or rom) into socket 7F aligning pin 14 to original

pin 12. Pins 1,2,27 and 28 will overhang the end

4. Connect Pin 28 to 7J pin 24 (+5v)

Pin  2 to 7D pin  5 (A12)

Pin 20 to 7J pin 12 (CE)

Pin 22 to 8E pin 13 (OE)

Pin 23 to 7D pin  3 (A11)

Pin 26 to 7D pin  7 (A13)

Pin 27 to 9N pin  6 (A14)    Switched by Menu

Pin  1 to 9N pin  7 (A15)    Switched by Menu

5. Bend up pin 21 on both graphics roms

6. Insert graph1 in 1H, graph2 into 1L

7. Connect pin 21 of each graphic rom to 9M pin 4

If all this has been done sucessfully, then power-up should give you a

menu listing the 4 available games. Select which game you wish to play

using left and right, press fire to start the game.

If you wish to add a button to return you to the menu, then you can extend

the on-board reset button to an unused button in the harness, or add an

extra button.